4b) Tabulka parametrů zboží určeného k plnění zakázky

HARDWARE PRO LOKÁLNÍ SBĚR DAT

(tabulka může být vyplněna v anglickém jazyce)

| Položka | Zadavatelem požadovaná hodnota | Uchazečem nabízená hodnota zboží určeného k plnění | Obchodní název dodávaného zboží |
| --- | --- | --- | --- |
| Processor | 2xCPU with benchmark at minimum 13 500 points: http://www.cpubenchmark.net/multi_cpu.html Hardware virtualization support |  |  |
| RAM | Minimum 128 GB DDR3 ECC Minimum 1600MHz frequency Support for up to 1TB ECC RAM |  |  |
| Storage interfaces | Support at minimum 2xSATA 3.0 6Gb/s AHCI, RAID 0,1,5,10 Support at minimum 2xSATA 2.0 3 Gb/s AHCI, RAID 0,1,5,10 Support at minimum 2xSATA 2.0 3 Gb/s SCU, RAID 0,1,5,10 |  |  |
| Disk | At minimum 1TB net disk space SATA 3.0, 7200rpm |  |  |
| DVD | 48x DVD ROM or RW |  |  |
| Connectivity | At minimum 2x 1Gb ethernet RJ 45, support for 10/100/1000BASE-T At minimum 4x 10Gb/s ethernet, 4x SFP+ slots, card in one PCIe x8 slot. Card must support: PCIe x3.0, RDMA (Remote DMA), TOE (TCP Offload Engine), jumbo frames, VMDq and PC-SIG SR-IOV foLarge receive offload (LRO), Linux.r Virtualized Environments, iSCSI remote Boot, PXE boot, Large send offload (LSO), Large receive offload (LRO), RSS (Receive Side Scaling), Linux. |  |  |
| Expansion slots | PCIe 3.0 At minimum 6x PCIe 3.0 x8 (or x16) signal lines At maximum 11x PCIe 3.0 x8 (or x16) signal lines. All devices in PCIe slots must be accessible from front doors when mounted in rack. Standard height in all slots must be supported. All slots must support full length cards. All cards must be mounted vertically. |  |  |
| USB | At minimum 8x USB 2.0 or 3.0. At minimum 4 USB ports accessible from front doors when mounted in rack. |  |  |
| Video | 1x VGA video accessible from front panel |  |  |
| Management support | Support for IPMI 2.0 on dedicated RJ45 ethernet port |  |  |
| OS compatibility | Linux X86 64bit, RedHat, SuSe |  |  |
| Electric connection | European slot |  |  |
| Chassis | Rack mount 19” Size 3 or 4 rack units (U) Delivery must contain whole rack mount kit including sliding-rails. |  |  |