Annex 1: Technical specification for the tendering:

High-energy diode-pumped nanosecond laser

| Wavelength | 1064 nm |
| --- | --- |
| Repetition rate | 100 Hz |
| Pulse energy | 1000 mJ |
| Energy stability | < 0,2 % |
| Pulse width at 1064 nm | 8-12 ns |
| Divergence (Half Angle): | < 0,5 mrad |
| Pointing Stability (RMS, 500 Pulse): | < ± 30 μrad |
| Beam Diameter (@1/e²); | ~ 8 mm |
| Temporal Jitter (RMS, 500 pulses): | < ± 1 ns |
| Diode Life Time: | 2 years or 4 x 10⁹ shots |
| Electrical Supply: | 240 VAC, 50/60 Hz |
| Cooling Water: | 8 l/min; 2-6 bar; < 15°C |
| Dim. Laser head IR (L x W x H): | 600 X 500 x 150 [mm] |
| Dim. Power Supply (L x W x H): | 6 RU (+6 RU for water/water cooler) |
| Weight Laser Head: | Less than 50 kg |
| Weight Power Supply: | ~ 50 kg (+ for water/water cooler) |
| Double pulse | As an option – to be upgradable |