The subject of the purchase is a metal evaporator with accessories whose basic purpose and mode of operation are described in Section 2.2 of the Invitation to submit bids. It must meet the technical conditions and include components listed in this table.

| No | Description and minimum specification of the subject of the purchase as defined by the Contracting Authority | Description and specification of the subject of the purchase offered by the bidder | Complies YES/NO |
| --- | --- | --- | --- |
| A | Evaporator |  |  |
| 1 | Mounting on DN 40 CF flange |  |  |
| 2 | Independent shutter for every evaporation position |  |  |
| 3 | At least 2 independent cells, filaments and flux monitors |  |  |
| 4 | Minimal working distance is 90 mm |  |  |
| 5 | Possibility to evaporate from metallic rods |  |  |
| 6 | Possibility to evaporate powder from crucible |  |  |
| 7 | Independent control unit for at least 2 evaporation positions (minimum 2 control units or similar solution) |  |  |
| 8 | Fully bakeable up to 200°C (or higher) |  |  |
| 9 | Independent z shift of material (rod, crucible) for every evaporation position. |  |  |
| 10 | Minimal evaporation temperature 120°C (or lower) |  |  |
| 11 | Maximal evaporation temperature 3000°C (or higher) |  |  |
| B | Control unit(s) |  |  |
| 12 | Possibility to control evaporation on device or remotely from the desktop pc. |  |  |
| 13 | Flux regulator |  |  |
| 14 | Emission current regulator |  |  |
| 15 | Possibility to regulate temperature of crucible |  |  |

(Bidders shall fill in the columns “Description and specification of the subject of the purchase offered by the bidder” and “Complies YES / NO”.)